1040826港貿作業代碼三十七、貨櫃種類之型式代碼，並自即日實施。

友財政部關務署　公告

發文日期：中華民國104年8月26日

發文字號：台關業字第1041019606號

附    件：如文

主    旨：公告新增「關港貿作業代碼」三十七、貨櫃種類之型式代碼，並自即日實施。

依    據：貨物通關自動化實施辦法第二十六條。

公告事項：三十七、貨櫃種類之型式代碼（如附件）。

署 長　饒 平

民國104年8月27日

附件：

三十七、貨櫃種類之型式代碼

本資料出處摘自：財政部關務署資訊網下載日期：2015年08月27日劉陽柳

三十七、貨櫃種類

主管機關：經濟部標準檢驗局

維護單位：財政部關務署（通關業務組、關務資訊組）

轉發布機關：財政部關務署（關港貿單一窗口）

編碼原則：

1.長度及屬性：長度為4碼，屬性為文數字。

2.碼意義說明：

第一碼：表示長度。

第二碼：表示寬度及高度。

第三碼：表示型式大分類。

第四碼：表示型式代碼或型式細分代碼。

使用本代碼之訊息：

N5101、N5103、N5165、N5166、N5167、N5201、N5202、N5203、N5262、N5301、NX401、NX5105

[附註]

1. 本代碼係參照國際標準組織代碼ISO 6346、經濟部標準檢驗局公布之CNS 3742「貨櫃之代碼、識別及標誌」編訂。

2. 貨櫃種類代碼第四碼，可用型式細分代碼或型式代碼來表示。例如：20呎長8呎寬8呎6吋高槽體貨櫃，若最低壓力分別為45KPa、150KPa或265KPa，貨櫃種類代碼第四碼可用型式細分代碼申報為22T0、22T1或22T2，或統一以型式代碼22TN(無危險性液體)申報。

貨櫃種類

（Container Type）

第一碼：表示長度。

| 代碼 | 貨櫃長度 |  |
| --- | --- | --- |
|  | 呎吋 | 毫米 |
| 1 | 10 | 2991 |
| 2 | 20 | 6068 |
| 3 | 30 | 9125 |
| 4 | 40 | 12192 |
| 5 | 其它 |  |
| A | - | 7150 |
| B | 24 | 7315 |
| C | 24 6 | 7430 |
| D | - | 7450 |
| E | - | 7820 |
| F | - | 8100 |
| G | 41 | 12500 |
| H | 43 | 13106 |
| K | - | 13600 |
| L | 45 | 13716 |
| M | 48 | 14630 |
| N | 49 | 14935 |
|  | - | 16154 |

第二碼：表示寬度及高度。

| 代碼 |  |  | 貨櫃高度 |
| --- | --- | --- | --- |
| 貨櫃寬度 |  |  |  |
| 2438毫米 (8呎) | >2438毫米 且≦2500毫米 | >2500毫米 | 呎吋 |
| 0 | - | - | 8 |
| 2 | C | L | 8 6 |
| 4 | D | M | 9 |
| 5 | E | N | 9 6 |
| 6 | F | P | ＞9 6 |
| 8 | - | - | 4 3 |
| 9 | - | - | ≦4 |

第三碼：表示型式大分類，第四碼：表示型式細分代碼。

| 第三碼 | 型式命名 | 型式 代碼 | 第四碼 | 主要特性 |  |
| --- | --- | --- | --- | --- | --- |
| G | 通用貨櫃 （無通風） | P | 0 | 一端或兩端有啟閉口 |  |
|  |  |  | 1 | 貨物空間之上部有被動式通氣口 |  |
|  |  |  | 2 | 一端或兩端有啟閉口，加上一側或兩次全部可開 |  |
|  |  |  | 3 | 一端或兩端有啟閉口，加上一側或兩次部分可開 |  |
|  |  |  | 4 | （未指定） |  |
| V | 通用貨櫃 （通風） | H | 0 | 無機械通風系統，通風口位於貨物空間之上、下部 |  |
|  |  |  | 1 | （未指定） |  |
|  |  |  | 2 | 機械通風系統，裝於內部 |  |
|  |  |  | 4 | 機械通風系統，裝於外部 |  |
| B | 散裝乾物貨櫃 |  |  |  |  |
|  | 非加壓裝卸，箱型 | U | 0 | 密閉式 |  |
|  |  |  | 1 | 氣閉式 |  |
|  |  |  | 2 | （未指定） |  |
|  | 加壓裝卸 | K | 3 | 水平卸貨，測試壓力150kPa |  |
|  |  |  | 4 | 水平卸貨，測試壓力265kPa |  |
|  |  |  | 5 | 傾斜卸貨，測試壓力150kPa |  |
|  |  |  | 6 | 傾斜卸貨，測試壓力265kPa |  |
|  |  |  | 7 | （未指定） |  |
| S | 專用貨櫃 | N | 0 | 活體動物裝載 |  |
|  |  |  | 1 | 汽車裝載 |  |
|  |  |  | 2 | 活魚裝載 |  |
|  |  |  | 3 | （未指定） |  |
| R | 保溼貨櫃 |  |  |  |  |
|  | 冷凍 | E | 0 | 機械式冷凍 |  |
|  | 冷凍及加熱 | T | 1 | 機械式冷凍及加熱 |  |
|  | 自備電力式冷凍/加熱 | S | 2 | 機械式冷凍 |  |
|  |  |  | 3 | 機械式冷凍及加熱 |  |
|  |  |  | 4 | （未指定） |  |
| H | 保溫貨櫃 |  |  |  |  |
|  | 可拆裝式冷凍與/或加熱裝置 | R | 0 | 裝於外部之可拆裝冷凍與/或加熱裝置，熱傳導係數k＝0.4W/（m2 x K） |  |
|  |  |  | 1 | 裝於內部之可拆裝冷凍與/或加熱裝置 |  |
|  |  |  | 2 | 裝於外部之可拆裝冷凍與/或加熱裝置，熱傳導係數k＝0.7W/（m2 x K） |  |
|  |  |  | 3 | （未指定） |  |
|  | 隔熱 | I | 5 | 隔熱：熱傳導係數k＝0.4W/（m2 x K） |  |
|  |  |  | 6 | 隔熱：熱傳導係數k＝0.7W/（m2 x K） |  |
|  |  |  | 7 | （未指定） |  |
| U | 開頂貨櫃 | T | 0 | 一端或兩端有啟閉口 |  |
|  |  |  | 1 | 一端或兩端有啟閉口，加上端框可移動頂材 |  |
|  |  |  | 2 | 一端或兩端有啟閉口，加上一側或兩側可開 |  |
|  |  |  | 3 | 一端或兩端有啟閉口，加上一側或兩側可開，加上端框可移動頂材 |  |
|  |  |  | 4 | 一端或兩端有啟閉口，加上一側部分可開，另一側全部可開 |  |
|  |  |  | 5 | 完全固定側邊與壁端（無櫃門） |  |
|  |  |  | 6 | （未指定） |  |
| P | 平台（貨櫃） |  |  |  |  |
|  | 附無櫃頂固定端壁無側壁平台貨櫃 | L | 0 | 平台（貨櫃） |  |
|  | 固定式 | F | 1 | 兩個完整與固定端壁 |  |
|  |  |  | 2 | 固定支柱，可自由站立或附可移動頂材 |  |
|  | 可摺端壁 | C | 3 | 可摺完整端壁結構 |  |
|  |  |  | 4 | 可摺支柱，可自由站立或附可移動頂材 |  |
|  | 完整上部結構之無側壁平台貨櫃 | S | 5 | 開頂，開端（骨架） |  |
|  |  |  | 6 | （未指定） |  |
| T | 槽體貨櫃 |  |  |  |  |
|  | 無危險性液體 | N | 0 | 最低壓力45kPa |  |
|  |  |  | 1 | 最低壓力150kPa |  |
|  |  |  | 2 | 最低壓力265kPa |  |
|  | 具危險性液體 | D | 3 | 最低壓力150kPa |  |
|  |  |  | 4 | 最低壓力265kPa |  |
|  |  |  | 5 | 最低壓力400kPa |  |
|  |  |  | 6 | 最低壓力600kPa |  |
|  | 氣體 | G | 7 | 最低壓力910kPa |  |
|  |  |  | 8 | 最低壓力2200kPa |  |
|  |  |  | 9 | 最低壓力（尚待決定） |  |
| A | 空運/海陸運貨櫃 |  | 0 |  |  |

日期：2015/08/27閱讀資料整理加值服務供分享歡迎指教    劉陽柳

第六節 ISO 6346 BIC CODES 貨櫃規格新代碼

The following is an overview of the most common codes:

| ISO Type Group |  | ISO Size Type |  |
| --- | --- | --- | --- |
| Code | Description | Code | Description |
| 20GP | GENERAL PURPOSE CONT. | 20G0 | GENERAL PURPOSE CONT. |
|  |  | 20G1 | GENERAL PURPOSE CONT. |
| 20HR | INSULATED CONTAINER | 20H0 | INSULATED CONTAINER |
| 20PF | FLAT (FIXED ENDS) | 20P1 | FLAT (FIXED ENDS) |
| 20TD | TANK CONTAINER | 20T3 | TANK CONTAINER |
|  |  | 20T4 | TANK CONTAINER |
|  |  | 20T5 | TANK CONTAINER |
|  |  | 20T6 | TANK CONTAINER |
| 20TG | TANK CONTAINER | 20T7 | TANK CONTAINER |
|  |  | 20T8 | TANK CONTAINER |
| 20TN | TANK CONTAINER | 20T0 | TANK CONTAINER |
|  |  | 20T1 | TANK CONTAINER |
|  |  | 20T2 | TANK CONTAINER |
| 22BU | BULK CONTAINER | 22B0 | BULK CONTAINER |
| 22GP | GENERAL PURPOSE CONT. | 22G0 | GENERAL PURPOSE CONT. |
|  |  | 22G1 | GENERAL PURPOSE CONT. |
| 22HR | INSULATED CONTAINER | 22H0 | INSULATED CONTAINER |
| 22PC | FLAT (COLLAPSIBLE) | 22P3 | FLAT (COLLAPSIBLE) |
|  |  | 22P8 | FLAT (COLL.FLUSH FOLDING) |
|  |  | 22P9 | FLAT (COLLAPSIBLE) |
| 22PF | FLAT (FIXED ENDS) | 22P1 | FLAT (FIXED ENDS) |
|  |  | 22P7 | FLAT (GENSET CARRIER) |
| 22RC | REEFER CONT.(NO FOOD) | 22R9 | REEFER CONT.(NO FOOD) |
| 22RS | BUILT-IN GEN. F. POWER SPLY OF REEF | 22R7 | BUILT-IN GEN. F. POWER SPLY OF REEF |
| 22RT | REEFER CONTAINER | 22R1 | REEFER CONTAINER |
| 22SN | NAMED CARGO CONTAINER | 22S1 | NAMED CARGO CONTAINER |
| 22TD | TANK CONTAINER | 22T3 | TANK CONTAINER |
|  |  | 22T4 | TANK CONTAINER |
|  |  | 22T5 | TANK CONTAINER |
|  |  | 22T6 | TANK CONTAINER |
| 22TG | TANK CONTAINER | 22T7 | TANK CONTAINER |
|  |  | 22T8 | TANK CONTAINER |
| 22TN | TANK CONTAINER | 22T0 | TANK CONTAINER |
|  |  | 22T1 | TANK CONTAINER |
|  |  | 22T2 | TANK CONTAINER |
| 22UP | HARDTOP CONTAINER | 22U6 | HARDTOP CONTAINER |
| 22UT | OPEN TOP CONTAINER | 22U1 | OPEN TOP CONTAINER |
| 22VH | VENTILATED CONTAINER | 22V0 | VENTILATED CONTAINER |
|  |  | 22V2 | VENTILATED CONTAINER |
|  |  | 22V3 | VENTILATED CONTAINER |
| 25GP | GP-CONTAINER OVER-HEIGHT | 25G0 | GP-CONTAINER OVER-HEIGHT |
| 26GP | GP-CONTAINER OVER-HEIGHT | 26G0 | GP-CONTAINER OVER-HEIGHT |
| 26HR | INSULATED CONTAINER | 26H0 | INSULATED CONTAINER |
| 28TG | TANK FOR GAS | 28T8 | TANK FOR GAS |
| 28UT | OPEN TOP (HALF HEIGHT) | 28U1 | OPEN TOP (HALF HEIGHT) |
| 28VH | VE-HALF-HEIGHT =1448 MM HEIGHT | 28V0 | VE-HALF-HEIGHT =1448 MM HEIGHT |
| 29PL | PLATFORM | 29P0 | PLATFORM |
| 2EGP | GEN. PURP. WITHOUT VENT WIDTH 2.5M | 2EG0 | HIGH CUBE CONT. (WIDTH 2.5M) |
| 42GP | GENERAL PURPOSE CONT. | 42G0 | GENERAL PURPOSE CONT. |
|  |  | 42G1 | GENERAL PURPOSE CONT. |
| 42HR | INSULATED CONTAINER | 42H0 | INSULATED CONTAINER |
| 42PC | FLAT (COLLAPSIBLE) | 42P3 | FLAT (COLLAPSIBLE) |
|  |  | 42P8 | FLAT (COLL.FLUSH FOLDING) |
|  |  | 42P9 | FLAT (COLLAPSIBLE) |
| 42PF | FLAT (FIXED ENDS) | 42P1 | FLAT (FIXED ENDS) |
| 42PS | FLAT (SPACE SAVER) | 42P6 | FLAT SPACE SAVER |
| 42RC | REEFER CONT.(NO FOOD) | 42R9 | REEFER CONT.(NO FOOD) |
| 42RS | REEFER CONT.(DIESEL GEN.) | 42R3 | REEFER CONT.(DIESEL GEN.) |
| 42RT | REEFER CONTAINER | 42R1 | REEFER CONTAINER |
| 42SN | NAMED CARGO CONTAINER | 42S1 | NAMED CARGO CONTAINER |
| 42TD | TANK CONTAINER | 42T5 | TANK CONTAINER |
|  |  | 42T6 | TANK CONTAINER |
| 42TG | TANK CONTAINER | 42T8 | TANK CONTAINER |
| 42TN | TANK CONTAINER | 42T2 | TANK CONTAINER |
| 42UP | HARDTOP CONTAINER | 42U6 | HARDTOP CONTAINER |
| 42UT | OPEN TOP CONTAINER | 42U1 | OPEN TOP CONTAINER |
| 45BK | BULK CONTAINER | 45B3 | BULK CONTAINER |
| 45GP | HIGH CUBE CONT. | 45G0 | HIGH CUBE CONT. |
|  |  | 45G1 | HIGH CUBE CONT. |
| 45PC | FLAT (COLLAPSIBLE) | 45P3 | FLAT (COLLAPSIBLE) |
|  |  | 45P8 | FLAT (COLL.FLUSH FOLDING) |
| 45RC | REEFER CONT.(NO FOOD) | 45R9 | REEFER CONT.(NO FOOD) |
| 45RT | REEFER HIGHCUBE CONTAINER | 45R1 | REEFER HIGHCUBE CONTAINER |
| 45UT | OPEN TOP CONTAINER | 45U1 | OPEN TOP CONTAINER |
| 45UP | HIGH CUBE HARDTOP CONT. | 45U6 | HIGH CUBE HARDTOP CONT. |
| 46HR | INSULATED CONTAINER | 46H0 | INSULATED CONTAINER |
| 48TG | TANK FOR GAS | 48T8 | TANK FOR GAS |
| 49PL | PLATFORM | 49P0 | PLATFORM |
| 4CGP | GP CONTAINER | 4CG0 | GP CONTAINER (WIDTH 2.5 M) |
| L0GP | HIGH CUBE CONT. | L0G1 | HIGH CUBE CONT. |
| L2GP | HIGH CUBE CONT. | L2G1 | HIGH CUBE CONT. |
| L5GP | HIGH CUBE CONT. | L5G1 | HIGH CUBE |

本資料出處摘自： 2013/05/17

http://en.wikipedia.org/wiki/ISO_6346

- Coding, Identifying, and Marking

The standard used to identify intermodal (shipping) containers is ISO 6346:1995. This standardized identification system is used to give each container a unique marking. The code can be further broken up into three parts an ISO 6346 (BIC) code, a size and type code, and additional optional markings.

Image Credit: The Geography of Transport Systems

http://www.globalspec.com/learnmore/material_handling_packaging_equipment/m

aterial_handling_equipment/iso_containers

本資料出處摘自： 2013/05/17

第七節 貨櫃種類代碼資料庫需求

日期:2013.05.20    中華貨物通關自動化協會常務理事  劉陽柳

一、開港單一窗口依據ISO6346 BIC CODE貨櫃種類代碼，新系統上線必須是用ISO 6346 BIC CODES以利商港服務費徵收正確性。

二、2013年5月17日海關會議中，交通部航政司資訊室主任褡答應會提供。也建議定時維護更新資料。

三、經查貨櫃上有標示貨櫃種類代碼，出口領空櫃時EIR貨櫃交接(吊櫃)單，只有極少數貨櫃站有標示新貨櫃種類新代碼。交通部應加強宣導。

Image Credit: The Geography of Transport Systems

四、進口D/O小提單上船公司全部使用舊代碼，交通部應加強宣導，使用新代碼。

五、建議報關即時系統資料增加貨櫃號碼及種類資料庫。

貨櫃號碼及種類資料

輸入貨櫃號碼馬上帶出貨櫃種類

回應時可覆蓋

進口報單、出口報單、轉運申請書系統

六、類型與尺寸的查詢參考表

貨櫃類型與尺寸的查詢參考表

http://www.npedi.com/edi/download/codeListContent.jsp?title=類型與尺寸的查詢參考表&url=../download/code_ctn_size_type_cn.html

|  | TW* 尺寸類型代碼 |  | ISO尺寸類型代碼 |  |
| --- | --- | --- | --- | --- |
| 類 型 | 箱 長 ft | 箱 高 ft in | 1984版本 | 1995版本 |
| DC | 20 | 0806 | 2200 | 22G0 |
| DC | 20 | 0806 | 2210 | 22G1 |
| DC | 20 | 0806 | 2280 | 22B0 |
| DC | 40 | 0806 | 4300 | 42G0 |
| DC | 40 | 0806 | 4310 | 42G1 |
| DC | 40 | 0806 | 4380 | 42B0 |
| DC | 45 | 0806 | 9210 | L2G1 |
| DC | 40 | 0906 | 9500 | L5G0 |
| DC | 45 | 0906 | 9510 | L5G1 |
| RF | 20 | 0806 | 2232 | 22R1 |
| RF | 40 | 0806 | 4330 | 42R0 |
| RF | 40 | 0806 | 4332 | 42R1 |
| RF | 40 | 0806 | 4340 | 42H0 |
| RH | 20 | 0906 | 2432 | 25R1 |
| RH | 40 | 0906 | 4532 | 45R1 |
| TK | 20 | 0806 | 2275 | 22T5 |
| HQ | 20 | 0906 | 2510 | 25G1 |
| HQ | 20 | 0906 | 2410 | 25G1 |
| HQ | 40 | 0906 | 4500 | 45G0 |
| HQ | 40 | 0906 | 4510 | 45G1 |
| HQ | 40 | 0906 | 4345 | 45H5 |
| OT | 20 | 0806 | 2251 | 22U1 |
| OT | 20 | 0806 | 2651 | 22U1 |
| OT | 40 | 0806 | 4351 | 42U1 |
| FC | 20 | 0806 | 2260 | 22P0 |
| FC | 20 | 0806 | 2263 | 22P3 |
| FC | 20 | 0806 | 2264 | 22P4 |
| FX | 20 | 0806 | 2261 | 22P1 |
| FC | 40 | 0806 | 4363 | 42P3 |
| FC | 40 | 0806 | 4364 | 42P4 |
| FX | 40 | 0806 | 4361 | 42P1 |
| OS | 20 | 0806 | 2065 | 20P5 |
| OS | 20 | 0806 | 2201 | 22G2 |
| OS | 20 | 0806 | 4301 | 42G2 |
| HH | 20 | 0403 | 2950 | 28U0 |
| PF | 20 | 0000 | 2960 | 29P0 |
| PF | 40 | 0000 | 4960 | 49P0 |
| HG | 40 | 0806 | 4312 | 42V7 |
| VE | 20 | 0806 | 2215 | 22V1 |
| VE | 40 | 0806 | 4320 | 42B0 |
| IN | 20 | 0806 | 222022B0 |  |
| IN | 40 | 0806 | 4320 | 42B0 |

第八節 台北港裝卸公司提供貨櫃英文數字代碼表

貨櫃英文數字代碼表台北港裝卸公司提供  103.07.07

| 貨櫃名稱 | 英文代碼 | 數字代碼 |
| --- | --- | --- |
| 二十呎普通櫃 | 22G1 | 2200 |
| 二十呎冷凍櫃 | 22R1 | 2230 |
| 二十呎油槽櫃 | 22T1 | 2270 |
| 二十呎帆布櫃 | 22U1 | 2250 |
| 二十呎平板櫃 | 22P1 | 2264 |
| 貨櫃名稱 | 英文代碼 | 數字代碼 |
| 四十呎普通櫃 | 42G1 | 4200 |
| 四十呎冷凍櫃 | 42R1 | 4230 |
| 四十呎油槽櫃 | 42T1 | 4270 |
| 四十呎帆布櫃 | 42U1 | 4250 |
| 四十呎平板櫃 | 42P1 | 4264 |
| 貨櫃名稱 | 英文代碼 | 數字代碼 |
| 四十呎普通高櫃 | 45G1 | 4500 |
| 四十呎冷凍高櫃 | 45R1 | 4530 |
| 四十呎油槽高櫃 | 45T1 | 4570 |
| 四十呎帆布高櫃 | 45U1 | 4550 |
| 四十呎平板高櫃 | 45P1 | 4564 |
| 四十五呎普通加長高櫃 | L5G1 | 9500 |